|  | when resistors have current through them they cool down - true or false |  | false - they are heated |
| --- | --- | --- | --- |
|  | Explain and give example of the meaning of the following ways of moving heat energy: Conduction |  | conduction: best through metal – atoms pass on vibrations |
|  | Explain and give example of the meaning of the following ways of moving heat energy: Convection |  | convection – caused by warm air rising - by moving particles |
|  | what is the formula for calculting energy efficiency? |  | eff = energy output / energy input |
|  | describe the main ways in which energy is transferred from a house to cooler surroundings and some common methods for reducing energy transfer |  | loft insulation, double glazing, draught excluders |
|  | Explain and give example of the meaning of the following ways of moving heat energy: Radiation |  | radiation – infra-red waves shine out like light - radiation transfers energy without any movement of matter and can travel through a vacuum |
|  | energy tends to flow from cool objects to hotter surroundings - true or false? |  | false - from hot objects to cooler surroundings |
|  | Draw energy flow diagrams for the following: |  |  |
|  | a) an electric hairdryer, |  | a) elect. à heat & kinetic & sound & light |
|  | b) gas fire |  | b) chemical potential à heat + light |
|  | c) an electric immersion heater. |  | c) elect à heat |
| - [] | Explain what a U-value is |  | Heat energy passing through a 1m2 surface with a temp difference of 1 oC every second |
| - [] | For the next questions you will need these U- values: |  |  |
| [] | Uninsulated brick wall: 1.5 W/m2/oC Double glazed window: 3.0 W/m2/oC |  |  |
| [] | Imagine your bedroom has an uninsulated cavity wall, which measures 3m by 6m. In the wall is double glazed window, which measures 1m by 0.5m. Calculate: |  |  |
| [] | a) The area of the glass part of the wall |  | 0.5m2 |
| [] | b) The area of the brick part of the wall. |  | 17.5m2 |
| [] | c) How much energy is lost per second through he walls when the temperature difference between outside and inside is 10 oC |  | 262.5 J |
| [] | d) How much energy is lost through the windows per second at these temperatures. |  | 15J |
| [] | e) Calculate the total energy losses |  | 277.5J |
| [] | f) Explain how double glazing works. |  | trapped air between two sheets of glass – limits convection and conduction losses |
|  | An electricity meter reads 632570 units in January and 632967 units in March – how much will the household be charged for electricity if the cost of one unit is 6p? |  | £23.82 |
|  | why is there demand for different levels of demand for electricity at different times of the day? |  | different demand due to different usage - eg. Tea break during adverts in Eastenders |
|  | How do solar panels work to provide a hot water supply for a house |  | they are placed on the roof - trapping sunlight and have cooler water trickling through them |
| - [] | write down the specific heat capacity formula. |  | energy transferred = mass x specific heat x temperature change |